ESP32-S3 Drone UAV

Flight Controller Project

All-in-One Quadcopter Firmware with WiFi Ground Control Station

Board:  ESP32-S3 DevKitC-1 N16R8 (16MB Flash / 8MB PSRAM)

WiFi:  AP Hotspot  —  SSID: DroneAP-ESP32S3 / PASS: drone1234

GCS URL:  http://192.168.4.1  (open in any browser)

Date:  July 13, 2026

# 1.  Project Overview

This project implements a complete quadcopter UAV (Unmanned Aerial Vehicle) flight controller using the ESP32-S3 DevKitC-1 N16R8 development board. The entire firmware is contained in a single C++ file (drone_uav.ino) and includes a full-featured Ground Control Station (GCS) web dashboard served directly from the ESP32 via a built-in WiFi Access Point. No external router or internet connection is required to control the drone.

## Key Highlights

- Single .ino file — entire flight controller + GCS in one file (43 KB)

- WiFi Hotspot — ESP32 creates its own network (no router needed)

- Web GCS at http://192.168.4.1 — works on any phone or laptop browser

- Dual-core FreeRTOS — Core 0: 250 Hz flight loop / Core 1: WiFi comms

- 9-axis PID stabilization — Roll, Pitch (angle mode) and Yaw (rate mode)

- Complementary filter — Gyroscope + Accelerometer sensor fusion at 250 Hz

- BMP280 barometer — real-time altitude and temperature readings

- Battery monitor — voltage + percentage via ADC voltage divider

- Live PID tuning — adjust all 9 gains from the GCS without reflashing

- Auto-disarm safety — motors cut if GCS signal lost for more than 2 seconds

- WebSocket telemetry — 10 Hz real-time data stream to GCS

# 2.  ESP32-S3 DevKitC-1 N16R8 — Specifications Used

| Feature | Specification | Used For in This Project |
| --- | --- | --- |
| CPU | Dual-core Xtensa LX7 @ 240 MHz | Core 0: 250 Hz flight loopCore 1: WiFi comms task |
| Flash | 16 MB | Firmware + embedded GCS HTML in PROGMEM |
| PSRAM | 8 MB OPI | JSON telemetry buffers |
| Wi-Fi | 802.11 b/g/n | AP hotspot — no router needed |
| Bluetooth | 5.0 LE | Future: BLE gamepad controller |
| LEDC PWM | Hardware PWM timers | 4x ESC 50 Hz 1000-2000 us output |
| ADC | 12-bit (0-4095) | Battery voltage monitor (GPIO 1) |
| I2C | 400 kHz fast-mode | MPU-6050 IMU + BMP280 barometer |
| GPIO | 45 pins available | ESC x4, I2C x2, ADC x1, LED x1 |

# 3.  System Features

## WiFi Access Point (Hotspot)

The ESP32-S3 creates its own WiFi network. No router or internet is needed. Connect any device to the hotspot and open the browser to access the GCS.

| Parameter | Value |
| --- | --- |
| SSID (Network Name) | DroneAP-ESP32S3 |
| Password | drone1234 |
| IP Address (GCS URL) | http://192.168.4.1 |
| WebSocket Endpoint | ws://192.168.4.1/ws |
| Max Clients | 4 simultaneous |
| WiFi Channel | 6 |
| Security | WPA2 |
| Auto-disarm timeout | 2 seconds on signal loss |

## PID Flight Stabilization

The flight controller uses three independent PID controllers for Roll, Pitch, and Yaw. Roll and Pitch operate in angle mode (setpoint = desired tilt angle in degrees). Yaw operates in rate mode (setpoint = desired rotation rate in deg/s). All gains can be updated live from the GCS without reflashing.

| Axis | Mode | Default Kp | Default Ki | Default Kd |
| --- | --- | --- | --- | --- |
| Roll | Angle (deg) | 1.40 | 0.03 | 18.0 |
| Pitch | Angle (deg) | 1.40 | 0.03 | 18.0 |
| Yaw | Rate (deg/s) | 3.50 | 0.02 | 0.0 |

# 4.  Hardware — Bill of Materials (BOM)

All components can be sourced locally in Pakistan (Lahore, Karachi, Islamabad) or from digilog.pk, mouser.com, and local electronics markets.

| # | Component | Specifications | Est. Price (PKR) |
| --- | --- | --- | --- |
| 1 | ESP32-S3 DevKitC-1 N16R8 | Main flight controller (you already have this) | 1,870 |
| 2 | F450 Quadcopter Frame | 450mm wheelbase, X-frame, includes landing gear | 2,000-2,500 |
| 3 | A2212 1000KV Brushless Motor x4 | 12A, 2S-3S LiPo compatible, standard 450 size | 3,000 (set) |
| 4 | 30A BLHELI_S ESC x4 | PWM input, 2-4S LiPo, BEC 5V/3A | 2,500 (set) |
| 5 | MPU-6050 IMU Module | I2C, 6-axis (gyro + accel), 3.3V logic | 300-500 |
| 6 | BMP280 Barometer Module | I2C, pressure + temperature, 3.3V logic | 200-400 |
| 7 | 3S LiPo Battery 2200mAh | 11.1V nominal, XT60 connector, 25C | 2,500-3,500 |
| 8 | 10x4.5 Propellers (2 pairs) | 1x CW pair + 1x CCW pair | 500 |
| 9 | XT60 Connector (M+F pair) | Battery + PDB connection | 200 |
| 10 | 47kΩ + 10kΩ Resistors | Battery voltage divider (through-hole) | 50 |
| 11 | Power Distribution Board | Optional, or solder ESC wires directly to LiPo | 400 |
| 12 | Dupont Wires & Pin Headers | For sensor connections (20cm female-female) | 200 |
|  | TOTAL (estimated) |  | 13,720-17,620 |

# 5.  Wiring Guide

NOTE: Use 3.3V for all sensors. Do NOT connect MPU-6050 or BMP280 to 5V.

## 5.1  I2C Sensors — MPU-6050 & BMP280

Both sensors share the same I2C bus (SDA/SCL lines).

| Signal | ESP32-S3 Pin | MPU-6050 Pin | BMP280 Pin | Notes |
| --- | --- | --- | --- | --- |
| SDA | GPIO 8 | SDA | SDA | Shared I2C data line |
| SCL | GPIO 9 | SCL | SCL | Shared I2C clock line |
| VCC | 3.3V | VCC | VCC | Both sensors use 3.3V ONLY |
| GND | GND | GND | GND | Common ground |
| AD0 | — | GND | — | Sets MPU-6050 address to 0x68 |
| SDO | — | — | GND | Sets BMP280 address to 0x76 |

## 5.2  ESC Motor Connections

| GPIO Pin | ESC Motor | Frame Position | Rotation | Propeller |
| --- | --- | --- | --- | --- |
| GPIO 4 (PIN_M1) | ESC 1 | Front-Right | Clockwise (CW) | Standard CW |
| GPIO 5 (PIN_M2) | ESC 2 | Rear-Right | Counter-Clockwise (CCW) | Pusher CCW |
| GPIO 6 (PIN_M3) | ESC 3 | Rear-Left | Clockwise (CW) | Standard CW |
| GPIO 7 (PIN_M4) | ESC 4 | Front-Left | Counter-Clockwise (CCW) | Pusher CCW |
| GND | All ESC GND | All motors | MUST share common GND | — |

NOTE: Only the ESC signal wire (orange/yellow) connects to GPIO. The power wires (red/black) go directly to battery via PDB.

## 5.3  Motor Layout — X-Frame (Top View)

FRONT

M4 (FL, CCW) ----+---- M1 (FR, CW)

|

M3 (RL, CW)  ----+---- M2 (RR, CCW)

REAR

CW  = Clockwise propellers (standard)

CCW = Counter-clockwise propellers (pusher)

## 5.4  Battery Voltage Divider

To monitor battery voltage safely, a resistor voltage divider scales the LiPo voltage (up to 12.6V for 3S) down to the ESP32 ADC input range (0-3.3V).

Battery (+) ---[47k Ohm]--- GPIO 1 (PIN_VBAT) ---[10k Ohm]--- GND

Divider ratio = (47k + 10k) / 10k = 5.7x

V_ADC = V_Battery / 5.7

At 12.6V battery: V_ADC = 2.21V  (within 0-3.3V ADC range)

# 6.  Software Setup

## 6.1  Install Arduino IDE

Download Arduino IDE 2.x from arduino.cc (recommended).

## 6.2  Install ESP32 Board Package

In Arduino IDE, go to: File → Preferences → Additional Boards Manager URLs and add:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

Then: Tools → Board → Boards Manager → search "esp32" → Install by Espressif Systems

## 6.3  Install Required Libraries

Go to: Sketch → Include Library → Manage Libraries

| Library | Author | How to Install |
| --- | --- | --- |
| ESPAsyncWebServer | me-no-dev | Search in Library Manager or install from GitHub ZIP |
| AsyncTCP | me-no-dev | Search in Library Manager or install from GitHub ZIP |
| ArduinoJson | Benoit Blanchon | Search "ArduinoJson" in Library Manager (v6 or v7) |

GitHub ZIP download URLs (Sketch → Include Library → Add .ZIP Library):

https://github.com/me-no-dev/ESPAsyncWebServer/archive/master.zip

https://github.com/me-no-dev/AsyncTCP/archive/master.zip

## 6.4  Arduino IDE Board Settings

| Setting | Value |
| --- | --- |
| Board | ESP32S3 Dev Module |
| Flash Size | 16MB (128Mb) |
| Partition Scheme | Default 4MB with spiffs |
| PSRAM | OPI PSRAM |
| CPU Frequency | 240MHz (WiFi) |
| Upload Speed | 921600 |
| USB Mode | Hardware CDC and JTAG |
| Port | Your COM port (check Device Manager) |

# 7.  Flash & First Boot

## Step 1 — Enter Download Mode

Hold the BOOT button, press and release RESET, then release BOOT. The ESP32-S3 is now ready to receive firmware.

## Step 2 — Upload

Open drone_uav/drone_uav.ino in Arduino IDE and click Upload.

## Step 3 — Serial Monitor

Open Serial Monitor at 115200 baud. Expected boot output:

+------------------------------------------+

|  ESP32-S3 DRONE UAV - Flight Controller  |

+------------------------------------------+

[Init] MPU-6050 ... OK

[Init] BMP280   ... OK

[Init] ESC arm pulse (2s)...

[Init] ESC ready

[Flight] Calibrating gyro (keep still ~1s)...

[Flight] Gyro offsets: GX=0.012 GY=-0.008 GZ=0.003

[WiFi] Hotspot SSID: DroneAP-ESP32S3  IP: 192.168.4.1

[HTTP] GCS at http://192.168.4.1

[Boot] System ready!

## Step 4 — Connect GCS

- On your phone or laptop, open WiFi settings

- Connect to: DroneAP-ESP32S3  (Password: drone1234)

- Open browser (Chrome, Safari, Firefox, Edge)

- Navigate to: http://192.168.4.1

- The GCS dashboard loads automatically

# 8.  GCS Dashboard Guide

The GCS dashboard is a complete web application served directly by the ESP32. It requires no installation and works on any modern browser including mobile devices.

## Dashboard Panels

| Panel | Description |
| --- | --- |
| Attitude Indicator | Animated artificial horizon. Shows real-time roll (bank) and pitch (nose up/down). The horizon drum rotates and shifts with drone attitude. |
| Heading / Compass | SVG compass rose with animated needle. Displays integrated yaw angle (0-360 degrees). Note: uses gyro integration only, not a magnetometer. |
| Altitude & Temp | Two vertical bar gauges. Altitude from BMP280 barometer (0-50m scale). Temperature from BMP280 sensor in degrees Celsius. |
| Motor Outputs | Four horizontal bars showing M1-M4 ESC pulse width in microseconds (1000=off, 2000=full). Updates 10 times per second. |
| Battery Monitor | Shows battery voltage (V) and percentage. Green above 40%, yellow 20-40%, red below 20%. Based on 3S LiPo range (9.6-12.6V). |
| Left Joystick | Throttle (up/down — HOLDS position on release) and Yaw/Rotate (left/right — springs back to center). |
| Right Joystick | Pitch (up/down) and Roll (left/right). Both axes spring back to center on release. |
| ARM / DISARM Button | Green ARM button to enable motors. Red DISARM to cut motors. Header pulses yellow when armed. |
| PID Tuning Panel | Expandable panel. Shows Kp/Ki/Kd for Roll, Pitch, Yaw. Click APPLY to send new values live to the drone. |
| Console Log | Real-time event log showing connection status, arm/disarm events, errors, and timestamps. |

## WebSocket Command Reference

The GCS communicates with the drone via WebSocket JSON messages:

| Command | JSON Payload | Effect |
| --- | --- | --- |
| Arm | {"cmd":"arm"} | Enables motors, sets armed=true |
| Disarm | {"cmd":"disarm"} | Cuts motors, resets throttle to idle |
| Control | {"cmd":"ctrl","thr":50,"roll":10,"pitch":-5,"yaw":0} | thr=0-100%, roll/pitch/yaw=-100% to +100% |
| PID | {"cmd":"pid","kp_r":1.4,"ki_r":0.03,"kd_r":18.0,...} | Updates PID gains live on the drone |
| Telemetry (drone->GCS) | {"roll":1.2,"pitch":-0.8,"yaw":45.0,"alt":1.5,...} | Sent 10 times/second by drone to GCS |

# 9.  PID Tuning Guide

NOTE: Always remove propellers before any tuning or testing.

## Tuning Procedure

- Flash the firmware with default gains

- ARM the drone on the bench (NO PROPS), apply ~20% throttle

- Tilt the drone by hand and observe the attitude indicator response

- Step 1: Set Ki=0, Kd=0. Increase Kp until you see fast oscillation, then reduce by 30%

- Step 2: Increase Kd gradually until oscillations dampen (typical range: 15-25)

- Step 3: Add small Ki (0.01-0.05) to correct any steady-state drift

- Repeat the same process for Pitch (usually identical to Roll)

- For Yaw: increase Kp_yaw until rotation response feels quick but not jittery

## Symptoms & Remedies

| Symptom | Likely Cause | Remedy |
| --- | --- | --- |
| Fast high-frequency oscillation | Kp too high | Reduce Kp by 20% |
| Slow hunting / low-freq oscillation | Kd too low | Increase Kd |
| Sluggish, slow response | Kp too low | Increase Kp gradually |
| Steady tilt / drift on hover | Ki too low | Increase Ki slightly |
| Oscillation gets worse over time | Ki too high | Reduce Ki (integral windup) |
| Overcorrects then oscillates | Kd too high | Reduce Kd by 20% |
| Yaw spins slowly on its own | Normal, no compass | Increase Kp_yaw or use rate Kd |

# 10.  Safety Checklist

NOTE: NEVER arm the drone with propellers fitted while indoors or near people.

## Before First Power-On

☐  Remove all propellers from motors

☐  Verify all motor wiring connections are secure

☐  Check I2C sensor wiring (3.3V, NOT 5V)

☐  Verify ESC signal wires match GPIO pins 4, 5, 6, 7

☐  Check voltage divider resistors are correct (47kΩ + 10kΩ)

## Before Each Flight

☐  Battery voltage > 12.0V (3S fully charged = 12.6V)

☐  All motor screws tight, propellers secured

☐  Frame arms and body locked, no loose parts

☐  Test ARM/DISARM cycle on bench first

☐  Verify motor direction matches CW/CCW layout before fitting props

☐  Test in open area away from people and obstacles

☐  Check weather: wind < 15 km/h for stable first flights

## During Flight

☐  Maintain visual line-of-sight at all times

☐  Land immediately if battery drops below 9.8V (10.5V recommended)

☐  Land immediately if attitude indicator shows unexpected behavior

☐  Keep first hover at 20-30 cm above soft ground

☐  Observe local regulations for UAV/drone operation

# 11.  Troubleshooting

| Problem | Likely Cause | Solution |
| --- | --- | --- |
| Serial shows "MPU-6050 FAIL" | Wrong I2C wiring or missing pull-ups | Verify SDA=GPIO8, SCL=GPIO9, 3.3V power, AD0 pulled LOW |
| Serial shows "BMP280 FAIL" | Wrong I2C address | Pull SDO pin of BMP280 to GND to set address 0x76 |
| Browser cannot reach 192.168.4.1 | Not connected to drone WiFi | Connect phone/laptop WiFi to DroneAP-ESP32S3 first |
| GCS shows OFFLINE | WebSocket connection failed | Refresh the page, check WiFi connection, check Serial for errors |
| Motors do not spin | Not armed or throttle too low | Click ARM in GCS, then slowly raise left joystick |
| ESC does not beep on power | No battery power to ESC | Check XT60 connector, check ESC power leads to PDB/battery |
| Upload fails in Arduino IDE | Not in download mode | Hold BOOT, press RESET, release BOOT, then Upload |
| Drone oscillates rapidly | Kp too high | Use PID panel in GCS to reduce Kp_roll and Kp_pitch by 20% |
| Drone drifts and leans | Gyro not calibrated or Kp too low | Keep drone still on boot for gyro calibration; increase Kp |
| Battery reads 0.00V in GCS | Voltage divider wiring issue | Check 47kΩ and 10kΩ resistors connected at GPIO 1 |
| Auto-disarm after 2 seconds | GCS command timeout safety | Normal behavior if WiFi signal drops; reconnect GCS |
| Library compile errors | Missing ESPAsyncWebServer or AsyncTCP | Install required libraries (see Section 6.3) |

# 12.  Project File Structure

esp32-drone-uav/

|-- drone_uav/

|   `-- drone_uav.ino    (43 KB — complete firmware + embedded GCS)

`-- README.md            (Full project documentation)

## drone_uav.ino — Internal Sections

| Section | Lines | Description |
| --- | --- | --- |
| 1. Includes | 1-10 | Arduino, Wire, WiFi, ESPAsyncWebServer, ArduinoJson, FreeRTOS headers |
| 2. Pin Definitions | 11-20 | GPIO assignments for ESCs, I2C, ADC, LED |
| 3. WiFi Config | 21-25 | SSID, password, IP address for AP hotspot |
| 4. MPU-6050 Registers | 26-35 | I2C register addresses and LSB scales |
| 5. BMP280 Registers | 36-45 | I2C register addresses for barometer |
| 6. ESC Config | 46-52 | PWM frequency, bit depth, min/max pulse widths |
| 7. PID Initial Gains | 53-62 | Default Kp/Ki/Kd values for Roll, Pitch, Yaw |
| 8. Flight Parameters | 63-75 | Max angles, throttle limits, loop rates, timeouts |
| 9. GCS HTML (PROGMEM) | 76-350 | Complete embedded web app: HTML/CSS/JavaScript |
| 10. Global State | 351-380 | DroneState struct, mutex, BMP280 calibration variables |
| 11. I2C Helpers | 381-395 | Wire.write/read/burst utility functions |
| 12. MPU-6050 Driver | 396-420 | Init and raw read (14 bytes: accel + temp + gyro) |
| 13. BMP280 Driver | 421-480 | Init, Bosch compensation formulas, altitude conversion |
| 14. ESC Motor Driver | 481-500 | LEDC PWM init, write, idle functions |
| 15. PID Controller | 501-520 | Struct with compute(), reset(), setGains() |
| 16. Flight Loop (Core 0) | 521-620 | Gyro calib, complementary filter, PID, motor mixing |
| 17. WebSocket Handler | 621-680 | arm/disarm/ctrl/pid command parsing |
| 18. Comms Task (Core 1) | 681-720 | Telemetry JSON build and WebSocket broadcast |
| 19. setup() | 721-780 | Sensor init, ESC arm pulse, WiFi AP, server routes |
| 20. loop() | 781-784 | Idle (all work in FreeRTOS tasks) |

# 13.  Firmware Architecture

## Dual-Core FreeRTOS Task Layout

Core 0 (Flight Task, Priority 5):

250 Hz loop (4 ms period)

- Read MPU-6050 (14 bytes I2C burst)

- Complementary filter: roll, pitch

- Integrate gyro Z for yaw accumulation

- PID compute: Roll, Pitch (angle mode), Yaw (rate mode)

- X-frame motor mixing matrix

- Write ESC PWM (LEDC)

- Every 10 loops: read BMP280 + battery ADC

- Signal timeout check (auto-disarm)

Core 1 (Comms Task, Priority 2):

10 Hz telemetry loop

- Snapshot DroneState (mutex-protected)

- Serialize to JSON

- WebSocket broadcast to all connected clients

- WebSocket client cleanup

AsyncWebServer (on Core 1, event-driven):

- GET /        -> Send GCS HTML (from PROGMEM)

- GET /health  -> "OK" health check

- WS  /ws      -> WebSocket upgrade, onWsEvent handler

## Sensor Fusion — Complementary Filter

The complementary filter combines gyroscope and accelerometer readings to produce stable attitude estimates free from gyro drift and accelerometer noise:

angle_roll  = 0.98 * (angle_roll  + gyro_x * dt) + 0.02 * accel_roll

angle_pitch = 0.98 * (angle_pitch + gyro_y * dt) + 0.02 * accel_pitch

0.98 = CF_ALPHA (gyroscope weight, high-pass filter)

0.02 = 1-CF_ALPHA (accelerometer weight, low-pass filter)

## X-Frame Motor Mixing Matrix

M1 (Front-Right, CW):  PWM = Throttle + Roll - Pitch + Yaw

M2 (Rear-Right,  CCW): PWM = Throttle + Roll + Pitch - Yaw

M3 (Rear-Left,   CW):  PWM = Throttle - Roll + Pitch + Yaw

M4 (Front-Left,  CCW): PWM = Throttle - Roll - Pitch - Yaw

PID output clamped to +-300 us per axis (PID_OUTPUT_LIM)

Final motor value clamped to 1050-1920 us (THROTTLE_IDLE to THROTTLE_MAX)